## THE NATIONAL UNIVERSITY OF LESOTHO

Department of Mathematics and Computer Science · Faculty of Science and Technology

CS3520 Computer Organisation and Architecture · AY 2026/2027

LIMO Group Project · Milestone M1 Submission

# LIMO Instruction Set Architecture

| Team | Visionary Architects | # Member | Student no. | GitHub |
| --- | --- | --- | --- | --- |
| Lab group | A & C | 1 Kopano Komanyane | 202321320 | GregoryHorsely |
| Release tag | m1 | 2 Mats’eliso Kheola | 202202304 | Matseliso20 |
| Commit | a83f21c | 3 Morapeli Seboka | 202401651 | Dahraps |
| Due | Sun 4 Oct 2026, 23:59 | 4 Liposo Ranoka | 202400088 | Liposo-Ranoka |
| Page limit | 6 pages | 5 Beleme Lenkoe | 202322644 | DonBelliiot |
|  |  | 6 Tumo Tshehla | 202400550 | Tshehlatumo-ops |
|  |  | 7 Thaane Moletsane | 202322687 | THMN123 |

Repository	https://github.com/THMN123/limo-Visionary-Architects.git

Assessed	Part I: ISA specification A1–A5 (40) and design-decision log A6 (20); defended individually at D1 (40).

## Team Charter and Set-up

Project board	https://github.com/users/THMN123/projects/1/views/2

Meeting rhythm	Tuesdays at 8pm, Thursdays at 7pm and Saturdays at 4pm.

Decision rule	weigh both sides (assessed pros and cons) and work on majority votes.

Definition of done	merged through a reviewed Pull Request, and project board updated

Branch and PR rules	Branches are named type/issue-number-short-name, e.g. feat/12-register-                table, fix/20-hex-encoding, docs/7-design-log. Each branch is linked to one issue on the project board. Work is merged into main only through a pull request that needs at least one approval from a teammate other than the author. main is protected: no direct pushes, and the m1 release  tag is placed on main only after the final pull request  is merged.

When a member is absent or not contributing

## ISA Specification

If a member is absent, the team first checks with them whether work can continue without them. If it can, we carry on and brief them on what was done with relevant documents .

| 2.1 Overview |  |  |
| --- | --- | --- |
|  | Specification | Rationale |
| Architecture style | 32-bit load–store; fixed-length 32-bit instructions (4 bytes); 32-bit word size. | Only explicit load and store instructions access memory, which keeps hardware complexity low and the pipeline straightforward and predictable. Fixed length simplifies instruction fetch and decode. |
| Register bank | 32 registers, r0–r31 (r0 hard-wired to zero): r0–r5 special-purpose (SPR), r6–r31 general-purpose (GPR); 5-bit register fields. | Enough room to compute without constantly spilling to memory. Three 5-bit fields use 15 bits, leaving 17 for opcode, funct and immediates. Matches RV32I. |
| Addressing modes | Register; immediate; base + offset (loads and stores); PC-relative (branches). | Base + offset reaches arrays and stack slots with a 12-bit signed offset; PC-relative branches need only a 13-bit offset. |
| Memory, endianness, alignment | Byte-addressed, little-endian, word-aligned; 32-bit addresses; memory size: | Every byte has a unique address; the least significant byte sits at the lowest address; 32-bit word accesses must use addresses divisible by 4 (address mod 4 = 0). |
| Instruction formats | R, I, S, B and J: RV32I-derived 32-bit layouts. | rs1, rs2 and rd sit in fixed positions across formats for simple decoding. The S immediate is split so register fields align with R-type; B encodes a 13-bit offset with bit 0 implicitly zero. |
| Departures from RV32I | Sesotho mnemonics; 10-instruction subset; register roles renumbered (r3 = pc, r4 = gp, r5 = fp, r6–r7 = arguments). | Sesotho mnemonics are a course requirement; the subset keeps M1 small while still covering arithmetic, logic, memory and branches. |
| 2.2 Register File |  |  |

| Number | Sesotho name | Alias | Type | Purpose and convention |
| --- | --- | --- | --- | --- |
| r0 | Letho | zero | SPR | Hard-wired to 0; writes are ignored |
| r1 | Khutlisa | ra | SPR | Return address (where to return after a function call) |
| r2 | Tshupo | sp | SPR | Stack pointer (top of the memory stack) |
| r3 | Sebali | pc | SPR | Program counter: address of the current instruction |
| r4 | Sesupi | gp | SPR | Global pointer (points to global variables) |
| r5 | Lesupa | fp | SPR | Frame pointer (tracks function memory frames) |
| r6–r7 | Thuso0–Thuso1 | a0–a1 | GPR | Argument registers: pass inputs and return results |
| r8–r15 | Nakoana0–Nakoana7 | t0–t7 | GPR | Temporaries: scratchpad calculations |
| r16–r27 | Polokelo0–Polokelo11 | s0–s11 | GPR | Saved registers: keep variables safe across calls |
| r28–r31 | TseHlahlamang0–3 | t8–t11 | GPR | Extra temporaries for additional scratch space |

### Instruction Formats

| funct7 | rs2 | rs1 | funct3 | rd | opcode |
| --- | --- | --- | --- | --- | --- |
| imm[11:0] |  | rs1 | funct3 | rd | opcode |
| imm[11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |
| imm[12¦10:5] | rs2 | rs1 | funct3 | imm[4:1¦11] | opcode |

31	25 24

##### R I S B

20 19

15 14

12 11	7  6	0

Immediate ranges: I: 12-bit signed, −2048 to +2047; S: 12-bit signed byte offset, −2048 to +2047; B (branch reach in bytes): 13-bit signed in multiples of 2, −4096 to +4094; J: 20-bit signed, shifted left by 1 (±1 MiB).

### Instruction Set and Sesotho Mnemonics

| Class | Mnemonic | Meaning | Syntax | Operation | Fmt | RV32I |
| --- | --- | --- | --- | --- | --- | --- |
| Arithmetic | kop | kopanya – to join/add | kop rd, rs1, rs2 | R[rd] ← R[rs1] + R[rs2] | R | ADD |
| Arithmetic | ts | tlosa – to subtract/remove | ts rd, rs1, rs2 | R[rd] ← R[rs1] − R[rs2] | R | SUB |
| Arithmetic | khh | kopanya hang-hang! – add immediately | khh rd, rs1, imm | R[rd] ← R[rs1] + imm | I | ADDI |
| Logic | le | le – and/with | le rd, rs1, rs2 | R[rd] ← R[rs1] & R[rs2] | R | AND |
| Logic | kp | kapa – or | kp rd, rs1, rs2 | R[rd] ← R[rs1] ¦ R[rs2] | R | OR |
| Logic | ng | ngatafala – multiply/shift up | ng rd, rs1, rs2 | R[rd] ← R[rs1] << R[rs2] | R | SLLI |
| Memory | lt | lata – to fetch/read | lt rd, imm(rs1) | R[rd] ← Mem[R[rs1] + imm] | I | LW |
| Memory | blk | boloka – to keep/store | blk rs2, imm(rs1) | Mem[R[rs1] + imm] ← R[rs2] | S | SW |
| Branch | hlk | ha e lekana – if it is equal | hlk rs1, rs2, offset | if R[rs1] = R[rs2]: PC ← PC + offset | B | BEQ |
| Branch | hft | ha e feta – if it passes/exceeds | hft rs1, rs2, offset | if R[rs1] > R[rs2]: PC ← PC + offset | B | BGT |

Lexical rules. Comments start with “# “ and run to the end of the line. Apostrophes are not permitted inside mnemonics; where a Sesotho word needs one, the letter is doubled instead (e.g. ts'ela becomes tss, not ts'), so every mnemonic is a plain letters-only token. LIMO is case sensitive.

### Encoding Worked Examples

| Instruction | Fmt | Field values | Binary (bit 31 … 0) | Hex |
| --- | --- | --- | --- | --- |
| kop r8, r6, r7 | R | funct7=0, rs2=7, rs1=6, funct3=0, rd=8 | 0000000 00111 00110 000 01000 0110011 | 0x00730433 |
| khh r8, r6, 15 | I | imm=15, rs1=6, funct3=0, rd=8 | 000000001111 00110 000 01000 0010011 | 0x00F30413 |
| blk r7, 4(r2) | S | imm=4, rs2=7, rs1=2, funct3=2 | 0000000 00111 00010 010 00100 0100011 | 0x00712223 |
| hlk r6, r7, +16 | B | imm=+16, rs2=7, rs1=6, funct3=0 | 0 000000 00111 00110 000 1000 0 1100011 | 0x00730863 |

## Design-Decision Log

- Our LIMO implements 32 architectural registers (RV32I-derived),  having 32 registers gives our processor enough space to handle calculations (like student grades) directly inside the CPU without constantly saving and loading data from memory. The downside is that each register address takes up 5 bits in an instruction (15 bits total for 3 registers), which leaves less room for constant numbers compared to a 16-register setup. In hardware, it also means using slightly larger circuits to compare register numbers when handling hazards. Every instruction in our LIMO processor is fixed at 32 bits total in length.

- Architectural: x0–x31 (x0 hardwired to zero), PC. These are visible to the ISA and must be preserved across exceptions. Microarchitectural only: IR (instruction register), IF/ID pipeline register, ID/EX, EX/MEM, MEM/WB, and any internal forwarding or temporary registers. The ISA must not specify these because they are implementation artefacts , that is, a different pipeline depth, out-of-order design, or single-cycle implementation would need different internal registers. Specifying them would freeze one microarchitecture and destroy compatibility across LIMO implementations. Only the programmer-visible state (registers, PC, memory) belongs in the ISA.

#### In our LIMO pipeline, branches resolve in the EX stage, where the ALU compares rs1 and rs2 and computes the target. With a classic 5-stage pipeline (IF, ID, EX, MEM, WB), this means two instructions fetched after the branch (in IF and ID) must be flushed when the branch is taken. Cost: up to 2 flushed instructions per taken branch, plus the branch penalty cycles. Extra hardware cost is small, that is, a comparator and target adder in EX. Resolving earlier in ID would need extra forwarding/compare hardware but reduce flushes to 1; resolving in MEM would need 3 flushes but almost no extra hardware, so EX is the balanced choice.

- Full forwarding cannot eliminate the load-use hazard because the loaded value is only available after the MEM stage, while the dependent instruction needs it at the start of its EX stage — one cycle too early. Cycle diagram (I1 = lw x1, 0(x2); I2 = add x3, x1, x4). I2's EX would need x1 at cycle 4, but I1 writes it at end of cycle 4 (WB). Forwarding MEM→EX gives it at cycle 5, so one stall bubble is mandatory

| Clock cycle | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| lt r8, 0(r2) [load] | IF | ID | EX | MEM | WB |
| kop r9, r8, r7 [dependent] | IF | ID | stall | Stall EX | MEM |

#### A flags register would create a new architectural state and a new hazard class: every ALU instruction would implicitly write flags, so a branch depending on flags would need to wait for the last flag-writing instruction even if that instruction's result register isn't otherwise needed. This adds false dependencies and complicates forwarding (flags become a second write port and a second forwarding path). It might save an encoding bit or two on branches, but the hazard cost and extra forwarding comparators outweigh that benefit. LIMO is better without flags.

- Immediate range breaks first. LIMO's I-type immediate is 12 bits (sign-extended, ±2048), S-type is 12 bits, B-type is 13 bits (±4 KiB branch reach), and J-type is 20 bits (±1 MiB jump reach). Register count (32) is generous and rarely the first limit. Branch reach becomes a problem only in very large functions. But immediate range bites almost immediately: array offsets, constants, and stack frame sizes beyond ±2048 require extra lui/addi sequences, inflating code size and instruction count long before register pressure or branch reach become critical

## Sample LIMO Programs

Program 1: Addition (Kopanya).  Adds two numbers stored in registers.

1	# Inputs: r6 = first number, r7 = second number; output: r8 = sum

2	kop	r8, r6, r7	# r8 = r6 + r7

Program 2: Store a constant (Boloka palo).  Stores a constant in memory

1	# Inputs: r6 = base value, r2 = stack pointer (Tshupo)

2	khh	r8, r6, 15	# r8 = r6 + 15 (I-format immediate add)

3	blk	r8, 4(r2)	# Mem[r2 + 4] = r8 (S-format store)

Program 3: Simple average (midpoint) of two numbers. Adds two numbers and returns their average

1	# Inputs: r6 = a, r7 = b; output: r8 = 2 * (a + b)

2	kop	r8, r6, r7	# r8 = a + b (R-format add)

3	khh	r9, r0, 1	# r9 = 1 (r0 is hard-wired to zero)

4	ng	r8, r8, r9	# r8 = r8 << 1 (shift left by r9 = 1)

## AI-Use Declaration

| Tool | Used for | How we checked or changed the output |
| --- | --- | --- |
| Claude | Formatting our own ISA notes (A1-A6) into the recommended template | We wrote the ISA design ourselves and used Claude only to fit it to the template. We checked every table against our notes then changed where there were errors and inconsistencies. |

We confirm that every member can explain every part of this submission and the code behind it.

Sprint Note

Sprint goal: LIMO ISA

##### Member	Role this sprint	Delivered (issue and PR numbers)

PRs reviewed

Kopano Komanyane	-	3	1

Thaane  Moletsane	-	1	1

Tumo Tsehla	-	2	1

Liposo Ranoka	-	3	1

Morapeli Seboka	-	1	1

Matseliso Kheola	-	1	1

Beleme Lenkoe	-	1	1

Not done, and why: [ done ]	Next sprint: [ m2 ]	Blockers: [ none }